SECTION 05 58 13 – METAL COLUMN COVERS

Items noted (in parenthesis and italic text) are optional

or variable depending on individual job conditions.

PART 1 - GENERAL

1.1 Section Includes

- Furnish all labor, materials, equipment and incidentals required to install column covers, in accordance with the contract drawings and details to insure a weather-tight system. Column Cover  supplier shall be responsible for the design of the Column Cover System, including, but not limited to, the anchorage details, sealant recommendations, and expansion/contraction provisions for system.

- Provide formed metal column covers and/or beam wraps as shown on drawings.  Contractor to coordinate requirements, quantities and sizes with structural drawings.

1.2 Related Sections

Not included as work or materials furnished in this section:

- Structural Steel, Section 05 _______.

- Light Gauge Metal Framing, Section 05 _______.

- Miscellaneous Metals, Section 05 _______.

- Sealant, Section 09 _______.

1.3 References

- The manufacturer will be a firm regularly engaged in the fabrication of custom aluminum columns cover systems with a minimum of 5 years experience in the production of systems as specified herein.

- The name of the manufacturer and/or installer must specifically be listed by the general contractor at the time of submission of the bid.

- No bidder of the column cover system will be accepted without the prior written approval of the architect based on submission of a mock-up illustrating the significant conditions of the project.

  - Scope of the mock-up must be closely coordinated with the architect.

  - Written approval of all bidders fulfilling these requirements shall be so noted by addendum.

1.4 Submittals

- Submit copies of manufacturer's standard guide specifications, standard detail drawings and installation procedures.

- Submit certified test reports verifying the performance requirements.

- Submit shop drawings showing small scale layout and elevations of the total project.  Details shall include attachment to the support system, include all information necessary for fabrication and recommendation for installation of column covers.  Indicate materials, sizes, thickness, fastenings and profiles.  Show details of weatherproofing at edges, termination, penetrations and connections to adjacent materials.

- Submit product samples as directed by the project architect representative of materials to be furnished.

1.5 Quality Assurance

- The installer shall be an experienced installation firm acceptable to the panel manufacturer. Written approval of any subcontract installer must be received by the architect prior to the installation of any materials.

- Reference to ASTM standards shall mean the latest edition of those standards.

- Any reference to specific manufacturers is not intended to restrict bidding but rather to establish a minimum standard of quality and offer clarification of panel profiles.  Manufacturers not listed must obtain the written approval of the architect a minimum of 7 days prior to the bid.  Such request for approval must be in accordance with paragraph 1.3 of this section and the general conditions.

1.7 Project Conditions

- Field Measurements Verify actual supporting and adjoining construction by field measurements before fabrication, and indicate recorded measurements on final shop drawings.  Coordinate construction to ensure that Column Covers fit properly to supporting and adjoining construction and coordinate schedule with construction progress to avoid delaying the work.

- Established dimensions:  where field measurements can not be made without delaying the work, guaranteed dimensions shall be provided to manufacturer to proceed with fabrication of Column Covers corresponding to the established dimensions.

PART 2 - PRODUCTS

2.1 Manufacturer

- Manufacturers: Subject to compliance with requirements, provide products manufactured by:

  - Alumafab Metal Sales, LLC Burnsville, MN.  www.alumafab.com,

Phone: (952)895-5700 Toll Free (800)544-2488 Fax: (952)895-1500

The Column Covers shall be installed in accordance with the details illustrated on the contract drawings.  The vertical and horizontal joints shall be sealed through the use of exposed sealants as indicated.  The column covers shall be positively attached to the building structure on the panel edges.

- AF-2000 – Aluminum Column Covers

- Alternate systems by other manufacturers/fabricators are to be submitted to the architect not less than 7 working days prior bid.

2.2 Panel Assembly

- The basic panel shall consist of custom fabricated, solid aluminum column covers, in thicknesses, lengths, widths and finish as illustrated on the contract drawings. Size: __(Diameter X Height)__

  - Each course to be fabricated in two or more vertical sections (typically halves) with butt joints.  Fold back ends of sheet metal to form a 1” wide return leg on the concealed side

  - AF2000 hairline joint utilizing key slot “Sur-Loc System”. Design and fabricate work to permit its installation on fully concealed mounting devices.

  - Bump forming of column cover will not be permitted unless prior written approval of Architect.

  - Form bent metal corners to the smallest radius possible without causing grain separation or otherwise damaging the work.

  - Provide straps, plates and brackets as required for support and anchorage of fabricated items.

- Covers shall be .125” aluminum 3003 alloy H14 temper.  Top and bottom rings shall be welded construction.  Horizontal butt joints with no reveals or caulk reveals shall be a minimum of ½” wide.

2.3 Finish

- Exterior surfaces of the metal panels shall be a fluoropolymer coating utilizing 70% KYNAR 500 resins with a total nominal dry film thickness of 1.0 mil.  Finish shall be applied in accordance with industry standards.

(For alternate finishes or more detailed specifications, refer to separate guide specification for insertion at this point.)

- One manufacturer's standard (custom) color shall be selected by the architect.

- All exposed surfaces shall be protected with a strippable coating.

- The strippable coating shall be compatible with the exterior finish.

- The strippable coating shall be removed by the erector immediately upon installation of the panels.

2.4 Accessories

- Fasteners

- All panels shall be positively attached to the structure through the use of concealed fasteners contained within the side joint of the assembly.  Installer to provide stainless steel screws and fasteners for aluminum column covers.

- No exposed fasteners will be accepted, unless noted otherwise.

- Fastener types and sizes shall be designed to resist negative & positive wind loads.

- Sealants

- All sealants shall be in accordance with the latest ASTM standards and shall comply with the sealant specifications of the contract documents.

- Subgirts & Clips

- Subgirts and clips shall be furnished as part of the scope of this work as noted on the contract drawings and as required to provide a complete Column Cover assembly.

PART 3 - EXECUTION

3.1 Examination

- Coordinate setting drawings, diagrams, templates, instructions, and directions for installation.

- Substructure shall be structurally sound and shall be free of defects detrimental to work and erected in accordance with established building tolerances.

3.2 Installation

- Comply with panel manufacturer's instruction for installation and erection of preformed metal panels.  Install in accordance with approved shop drawings.

- Anchor component parts securely in place, providing for necessary thermal and structural movement.

- Examine structure and surfaces for defects that would prevent proper installation of the wall system.  Report any discrepancies to the architect and contractor.  Do not proceed until the remedial work has been completed.

- All fasteners shall be installed in accordance with the materials specification above.

- Covers to be secured to substrate by use of formed aluminum clips utilizing angle, adjustable angle or zee shapes.  Number of clips required to be determined by manufacturer’s standard practice.  Quantity to be approved by Architect.

- Isolate dissimilar materials in accordance with acceptable industry practices and as recommended by panel manufacturer.

3.3 Cleaning

- Touch-up damaged finishes.

- Cleaning and maintenance per maintenance instructions.

3.4 Damaged Column Covers

- Repair or replace any and all Column Covers and trim that have been damaged upon determination of responsibility.

ALTERNATE SPECIFICATION OPTIONS

At specifier's option, the following alternatives may

be selected in lieu of the above standard options.

1. For most applications, Aluma-Fab recommends the use of .125" aluminum plate, however alternate material thickness' are also available.  If an alternate material thickness is preferred:

*Delete the word ".125" " in paragraph 2.2/B and substitute ".090".

2. The above specifications are for materials, which are coated with KYNAR or some other type of paint finish.  If the materials are to be anodized, make the modifications noted below.  Specifier should be aware that the welded corners and other areas of the panel will accept the anodized finish differently that the aluminum plate.

*Delete the words "3003 alloy, H14 temper"" in paragraph 2.2/B and substitute "5005 AQ alloy, H32 temper."

*Delete paragraph 2.3/ A and B and substitute the separate guide specification following.

- Class I, Clear Anodic Finish: AA-M12C22A41 (Mechanical finish: nonspecular as fabricated; Chemical Finish: etched, medium matte; Anodic Coating: Architectural   Class I, clear coating 0.018 mm or thicker) complying with AAMA 607.1.

Or

- Class I, Color Anodic Finish: AA-M12C22A42/A44 (Mechanical finish: nonspecular as fabricated; Chemical Finish: etched, medium matte; Anodic Coating: Architectural Class I integrally colored or electrolytically deposited color coating 0.018 mm or thicker) complying with AAMA 606.1 or AAMA 608.1.

- Color: As selected by Architect from the full range of industry colors and color densities.

- Color: Match Architect’s sample.